Lecture – 1 | Communication Network

Infrastructure of Communication Network

- Electrons and photos as communication medium.

- Links: fiber, copper, satellite.

- Switches: mechanical, electronic, optical.

- Protocols: TCP/UP, ATM, Ethernet, SNA.

- Functionalities: routing, error control, congestion control.

- Applications: FTP, WEB, …

Types of Networks

- Geographical distance

  - LAN

  - MAN

  - WAN

- Information Type

  - Data networks

  - Telecommunication networks

- Application Type

  - Special Purpose Networks

    - Airline reservation network

    - Banking network

    - Payment gateway network

    - Telephony

  - General purpose network

    - Internet

Internet Engineering Task Force (IETF): The IETF is the

protocol engineering and development arm of the Internet.

Subdivided into many working groups, which specify

Request For Comments or RFCs.

IRTF (Internet Research Task Force): The Internet

Research Task Force is composed of a number of focused,

long-term and small Research Groups.

Internet Architecture Board (IAB): The IAB is responsible

for defining the overall architecture of the Internet, providing

guidance and broad direction to the IETF.

The Internet Engineering Steering Group (IESG): The

IESG is responsible for technical management of IETF

activities and the Internet standards process. Composed of

the Area Directors of the IETF working groups.

Classification of Communication Networks

- Broadcast communication network

Information transmitted by any node is received by every other node in the network. Usually in LANs (Ethernet).

Problem is accessing all nodes to the shared communication medium (Multiple Access Problem).

- Switched communication network

Information is transmitted to a sub-set of designated nodes. E.g. WANs (Internet, Telephony Network).

But how to forward information to intended node(s)? This is done by special nodes (routers, switches) running routing protocols.

  - Circuit-Switched communication network

It runs in three phases - Circuit Establishment, Data transfer, Circuit termination. If circuit is not available, then it returns busy signal. E.g. Telephone networks, ISDN (Integrated Services Digital Networks).

  - Packet-switched communication network

    - Datagram network

    - Virtual circuit network