Module 4

Taxonomy of Attack Classes

- Non-invasive attack

  - Lowest cost

  - No knowledge of inner workings of target

  - No physical tampering

- Semi-invasive attack

  - Intermediate cost

  - Some knowledge of inner working of target

  - Minimal physical tampering required

- Invasive attack

  - High cost

  - Full picture of inner workings of target

  - Best chance of compromising target

Classification of threats

- Skilled outsider

  - Exploit existing weaknesses

  - Minimal equipment sophistication

  - Black-box understanding of target system

- Knowledgeable insider

  - Advanced education and technical expertise

  - Moderate equipment sophistication

  - Some functional knowledge of target system

- Funded organization

  - Highest education and technical expertise

  - High equipment sophistication

  - High-complete functional knowledge of target

Levels of Security

- Level 1

  - Bare minimum required protection

  - Minimal defense against glitching and tampering

- Level 2

  - Some tamper proofing

  - Some defense against glitch attacks

- Level 3

  - Passive system lock-outs

  - Passive tamper proofing

- Level 4

  - Active system lock-outs

  - Active tamper detection

Design for security

- Cost of a security breach

  - Loss of customers and reputation

  - Fines from government

  - Loss of bottom line

- Value of secured data to attacker

  - Commercial value

  - Strategic value

  - Profitability

- Cost of security implementations

  - Price increase

  - Area and complexity increase

  - Power consumption increase

Overview of non-invasive attacks

- Black Box Attacks

  - Brute Force Attack

    - Memory verify guessing

    - Cryptographic key guessing

    - Cyphertext-to-Plaintext Guessing

  - Software Attack/Exploits

    - Undocumented functions

    - Security function flaws

    - Test interface flaws

  - Data Remanence

    - Lower temperature to -20C or less

    - Volatile memory retains data

    - Read volatile memory contents

- Side Channel Attacks

  - A type of security exploit that relies on information gained from the physical implementation of a computer system, rather than weaknesses in the implemented algorithm itself.

  - This attack leverages indirect information such as timing data, power consumption, electromagnetic leaks, or even sound to extract sensitive information like cryptographic keys.

  - Cache Attack: This attack monitors cache accesses made by the victim in a shared physical system, such as a virtualized environment or cloud service. By observing these accesses, an attacker can deduce the encryption key.

  - Timing Attack: This attack measures the time taken to perform cryptographic operations. Variations in timing can reveal the entire secret key through statistical analysis.

  - Power-Monitoring Attack: This attack observes the power consumption of a hardware device during computation. Variations in power usage can provide detailed information about the operations being performed.

  - Electromagnetic Attack: This attack analyzes electromagnetic radiation emitted by a device. Similar to power analysis, it can infer cryptographic keys or other sensitive information.

  - Acoustic Cryptanalysis: This attack exploits sound produced during computation. For example, the noise generated by a CPU can reveal information about the operations being performed.

- Fault Injection Attacks

  - An attack that intentionally causes errors in a system to compromise the security of the system.

  - Clock Glitching

    - Burst of double clock speed – timing critical

    - Requires knowledge gained from side-channel attack

    - Prevent flip-flops from latching correct data

    - Prevent security fuses from setting properly

    - Could cause skipping instructions

  - Voltage Glitching

    - Burst of high or low voltage – timing critical

    - Requires knowledge gained from side-channel attack

    - Force VDD < VTH

    - Prevent security fuses from setting properly

    - Change control logic outputs

    - Change memory amplifier output

Overview of semi-invasive attacks

- Backside Decapsulation

  - Backside Imaging

    - Substrate penetrated by infrared light

    - Transistor layout is visible through substrate

    - Reverse engineering of block-level functionality

  - Laser Scanning

    - Optical Beam Induced Current (Unpowered IC)

    - Light-Induced Voltage Alteration (Powered IC)

    - See memory structures and read stored values

  - Reverse Engineering

    - Determine size of data bus

    - Determine location of control logic

    - Determine location of security logic

- Fault Injection Attacks

  - Local Heating

    - High power laser is used to selectively heat small areas

    - Hot enough to change VTH but not hot enough to damage

    - Trial and error with location is used to determine glitches

  - Flash Glitching

    - Magnified camera flash can cause mass glitching

    - Tinfoil masks created to cause selective glitching

    - Trial and error with location and timing is used to determine glitches

  - Laser Glitching

    - Infrared laser is used to selectively glitch small areas

    - Trial and error with location and timing is used to determine glitches

    - Process is more precise than Flash Glitching

Overview of invasive attacks

- Reverse Engineering

  - Decapsulation

    - Use of acids to remove layers one by one

    - Provide physical access to all sections of IC

    - Provide knowledge about design of IC

  - Layout Reconstruction

    - Image each layer before removing it

    - Build netlist from all images

    - Reverse engineer all functions of IC

  - Memory Extraction

    - Read contents of ROM with reconstruction

    - Scan contents of SRAM

    - Scan contents of EEPROM/FLASH

- IC Modification

  - Laser Cutting

    - Not completely destructive

    - Selective exposure of lower layers

    - Selectively disconnect nets

  - Test Point Creation

    - Cut test points into IC

    - More spots for micro probing below top layer

    - See more signals on more nets

  - Wire Bonding

    - Use laser cutting to expose net

    - Cut test point for bonding target

    - Modify circuit paths as needed

- Micro Probing

  - Eavesdropping

    - Listen to control lines

    - Listen to data bus

    - Full bypass of all protections

  - Signal Injection

    - Insert control signals

    - Modify memory contents

    - Forcefully bypass security controls

  - Fault Injection

    - High voltage between two probes

    - Destroy transistors

    - Destroy traces