Physical Attacks: Technical Application

| Attack Vectors | Example | Countermeasures | Cost | Usability | Security |
| --- | --- | --- | --- | --- | --- |
| Physical tampering | Removing screws, open enclosure | Use a metal mesh connected to a tamper detecting controller. | Medium, as it would require mid-level-sophisticated hw/sw resource | Low/Medium, as it may slightly hinder authorized maintenance | High, as it prevents simple extraction while providing detection |
| Chip decapping for SoC extraction | Removing chip and read internal flash or extract keys | Keeping keys in a dedicated secure element that never exposes raw keys. Also, using encrypted firmware images signed by device vendor itself is encouraged. | High, as it requires well setup and highly sophisticated hw/sw, human resources as well | Low, as it is transparent to users | High, as it increases complexity. |
| Port abuse/ JTAG access | Access exposed debug headers to dump memory and/or patch firmware | Seal off debug in production, require physical key for debug for authorized service only. | Low, as it requires very low effort and sophistication to implement | Medium, as it restricts field debugging | High, as it blocks a common and low cost attack path |
| Wireless physical promity attacks | Sniff or relay commands to control device | Use authenticated encrypted links, re-pairing with physical confirmation (like a button or something). | Low, as this implementation can be handled softly (software wise) | Low, as it may require user action for pairing | High, as it protects over-the-air controls |
| Fault injection | Inject voltage glitch to skip secure boot signature checks or enable bootloader | Implement secure boot that verifies signatures in hardware backed root or trust. | Medium, as most of the implementation relies on software/sw part of the measure | Medium, as it may require robust power design | High, as it prevents bypassing of firmware validation |